HFSS-Based Compact Modeling of Cascaded RDL-TSV Interconnects Using Equivalent Circuits and Transition Correction

Zhang-Zhe Feng
School of Electronics and Information (School of IC Science and Engineering) 
Hangzhou Dianzi University
Hangzhou, China
246040048@hdu.edu.cn

Absalom El-Shamir Ezugwu
Unit for Data Science and Computing
North-West University
Potchefstroom, South Africa
absalom.ezugwu@nwu.ac.za
Wen-Sheng Zhao
School of Electronics and Information (School of IC Science and Engineering)
Hangzhou Dianzi University
Hangzhou, China
wshzhao@hdu.edu.cn

Hairong Wang
School of Computer Science and Applied Mathematics
University of the Witwatersrand
Johannesburg, South Africa
hairongwng@gmail.com
Da-Wei Wang
School of Electronics and Information (School of IC Science and Engineering)
Hangzhou Dianzi University
Hangzhou, China
davidw.zoeq@hdu.edu.cn

Wen-Sheng Zhao
School of Electronics and Information (School of IC Science and Engineering)
Hangzhou Dianzi University
Hangzhou, China
wshzhao@hdu.edu.cn

Abstract-This paper presents an compact modeling method for cascaded redistribution line (RDL) and through-silicon via (TSV) interconnect structures. Scalable equivalent-circuit models are first built for individual RDL and TSV devices, and an RDL-TSV Transition Model is inserted at each junction to capture local parasitic effects. To reduce parameter ambiguity in long cascades, shared seven-parameter optimization is followed by a twelve-head neural network for position-dependent correction. For the TSV-RDL3 validation case, the model achieves 3.1% average NMSE using 120 training samples and 58 test samples. The resulting model is compact, interpretable, SPICE-compatible, and suitable for PDK-oriented applications.

Keywords-RDL, TSV, equivalent circuit, compact model, transition model

Introduction

Three-dimensional (3-D) integration enables high-density heterogeneous systems by vertically stacking dies and connecting them through redistribution layers (RDLs) and through-silicon vias (TSVs). As RDL-TSV interconnect chains become longer, their frequency response is no longer determined by isolated RDL or TSV sections alone. Transition discontinuities, repeated cascade errors, and phase accumulation jointly affect insertion loss, return loss, and delay.

Full-wave electromagnetic simulation can accurately characterize these effects, but directly simulating every full-chain geometry in HFSS is too expensive for design-space exploration, compact-model library construction, and PDK-oriented development. An accurate compact model with clear circuit meaning and SPICE compatibility is therefore needed.

Existing frequency-domain models usually follow two routes. Direct neural-network S-parameter surrogates can fit sampled responses, but they have limited physical interpretability and are difficult to reuse in circuit simulation. Cascading compact models of individual devices is more interpretable, but it ignores parasitic effects introduced at RDL/TSV transitions. For long chains, these small interface errors can accumulate into significant full-structure deviations.

This work proposes a compact modeling flow that combines HFSS-derived equivalent-circuit models for individual RDL and TSV sections with a seven-parameter pi-type RDL-TSV Transition Model. A shared-to-multi-head neural network maps device geometry to transition-model parameters for the twelve junctions in the complete chain. The resulting model keeps the physical interpretability of equivalent circuits while improving full-chain accuracy over a direct cascade of isolated single-device models.

Modeling Method

Overall Modeling Flow

Fig. 1 summarizes the overall modeling flow for the cascaded RDL-TSV structure. First, equivalent-circuit models are established for individual RDL and TSV devices. Based on the selected circuit topologies, the element values are extracted from HFSS-simulated S-parameters. For each circuit element, an MLP maps the physical device parameters to the corresponding element value, producing scalable compact models for individual RDL and TSV sections.

After the scalable RDL and TSV models are constructed, they are used to replace the corresponding sections in the complete interconnect structure. At each RDL/TSV junction, a pi-type transition model is inserted to characterize coupling, discontinuity, and loss caused by the physical transition between the two device types. The complete compact model is then formed by cascading multiple RDL models, TSV models, and RDL-TSV Transition Models according to the physical order of the interconnect chain.

For a chain with many RDL/TSV junctions, independently optimizing all transition models introduces many circuit parameters and can lead to non-unique solutions. Therefore, a two-step training strategy is used. The transition model is first optimized in a shared form, where the same circuit parameters are repeated at all connection positions. The shared optimized targets are used to train an initial parameter network. The network is then expanded into a multi-head model with twelve output heads and trained against the full-chain S-parameter response. This strategy captures position-dependent interface behavior while reducing parameter ambiguity.

RDL Equivalent-Circuit Model and Parameter Extraction Method

Zc γThe RDL model is built from the equivalent-circuit topology reported in [2], as shown in Fig. 2. This circuit accurately represents the frequency response of an RDL section, and its element values can be extracted from S-parameters. For each HFSS sample, the RDL S-parameters are converted into an ABCD matrix. The characteristic impedance Zc and propagation constant gamma are then calculated as

| ABCD= | (1) |
| --- | --- |
| Zc=BC | (2) |
| γ=acos⁡((A+D)/2)l | (3) |
|  | (4) |

ZcγUsing Zc and gamma, the per-unit-length RLGC curves are obtained as

| R=Re⁡(γZc) | (5) |
| --- | --- |
| L=Im⁡(γZc)ω | (6) |
| G=Re⁡γZc | (7) |
| C=Im⁡(γ/Zc)ω | (8) |

According to the equivalent-circuit formulation, when the circuit accurately represents the RDL device response, the frequency-dependent resistance, inductance, conductance, and capacitance implied by the circuit parameters must match the RLGC curves extracted from HFSS. Therefore, the following relationships should be satisfied:

| R12R2+R1R22+ω2R1L22R1R2)2ω2L22+ω2L32R3R32+ω2L32≈R(ω) | (9) |
| --- | --- |
| L1+R12L2R1R2)2ω2L22+L3R32R32+ω2L32≈L(ω) | (10) |
| ω2R4C121+ω2R42(C1+C2)2≈G(ω) | (11) |
| C1+ω2R42C12C2+ω2R42C22C11+ω2R42(C1+C2)2≈C(ω) | (12) |

The circuit parameters can then be sequentially extracted from the RLGC curves using the following formulas:

| R3=R(ω3) | (13) |
| --- | --- |
| R1=R(ω2)-R3 | (14) |
| R2=R1R(ω1)R1-R(ω1) | (15) |
| L1=L(ω2) | (16) |
| L3=L(ω3) | (17) |
| L2=L(ω1)-L1-L3R1+R2R12 | (18) |
| C1=C(ω1) | (19) |
| C2=C1C(ω2)C1-C(ω2) | (20) |
| R4=C12G(ω2)(C1+C2)2 | (21) |

ω1ω2ω3ω1=108×2πω2=1011×2πω3=2×1010×2πHere, omega1 is selected at the lowest frequency, omega3 is selected at the highest frequency, and omega2 is selected at an intermediate frequency. In this work, these angular frequencies are set to 0.1 x 10^9 x 2pi rad/s, 20 x 10^9 x 2pi rad/s, and 100 x 10^9 x 2pi rad/s, respectively.

TSV Equivalent-Circuit Model and Parameter Extraction Method

RTSVLTSVCoxCSiRSiThe TSV model is based on the equivalent-circuit model proposed in [3], as shown in Fig. 3. This circuit includes the TSV conductor resistance R_TSV and inductance L_TSV, oxide capacitance C_ox, silicon-substrate capacitance C_si, and substrate loss resistance R_si. To simplify parameter extraction, the original equivalent circuit is transformed into the alternative circuit shown in Fig. 4. Although the two circuits have different topologies, they provide the same frequency response and are therefore equivalent for S-parameter modeling.

CoxCSiRSiωωRTSVωLTSVωSimilar to the RDL extraction, the TSV S-parameters are converted into ABCD matrices and used to derive RLGC curves. The parameters C_ox, C_si, and R_si are extracted from the G(omega) and C(omega) curves using the same procedure as the RDL shunt-branch extraction in (7)-(9). R_TSV is taken from the low-frequency value of R(omega), while L_TSV is taken from the high-frequency value of L(omega).

RDL–TSV Transition Model

The proposed RDL-TSV Transition Model is shown in Fig. 5. This model is inserted at each RDL/TSV junction to compensate for parasitic effects that are not captured by directly cascading isolated RDL and TSV compact models. It is implemented as a seven-parameter pi-type equivalent circuit. The shunt capacitance-resistance branches model local capacitive coupling and dielectric or substrate loss near both sides of the transition, while the series resistance and inductance describe additional transition loss, current crowding, and local current-path inductance introduced by the cascade discontinuity.

Validation and Discussion

To verify the effectiveness of the proposed modeling method, the TSV-RDL3 case reported in [2] is selected as the validation example, as shown in Fig. 7. The structure consists of four top-metal-layer RDL sections, three bottom-metal-layer RDL sections, and six TSV sections. Therefore, the complete interconnect chain contains thirteen device sections and twelve RDL/TSV junctions.

TABLE I

Parameters of RDL and TSV (Unit: um)

| Device | Parameter | Max | Min |
| --- | --- | --- | --- |
| RDL | Length | 100 | 700 |
|  | Width | 10 | 30 |
|  | Thickness | 2 | 5 |
|  | Pitch | 40 | 60 |
|  | Substrate height | 56 | 106 |
| TSV | Height | 50 | 100 |
|  | Diameter | 10 | 30 |
|  | Pitch | 40 | 60 |

For the RDL and TSV single-device models, 400 geometry samples are generated in their parameter spaces using Latin hypercube sampling, and their S-parameters are obtained by HFSS simulation. The equivalent-circuit parameters are extracted using the models and formulas described in Sections II-B and II-C. For each circuit element parameter, an MLP with two 20-neuron hidden layers is trained to map the physical device parameters to the corresponding element value. The trained RDL and TSV models achieve average NMSE values of 0.04% and 0.02%, respectively, indicating that the single-device compact models can accurately reproduce the HFSS responses.

After the individual RDL and TSV device models are established, these compact models are used to replace the corresponding RDL and TSV sections in the complete structure. The replaced device sections are then connected by the RDL-TSV Transition Model, forming a full-chain circuit composed of RDL models, transition models, and TSV models.

For the RDL-TSV Transition Model, 120 samples are used for training and 58 samples are used for testing. The initial parameter network uses a 30 x 30 x 20 MLP structure. The final model is expanded into a multi-head network with twelve output heads, where each head predicts the transition-model parameters for one RDL/TSV junction. Using the S-parameters of the complete cascaded structure as the target, the final full-chain model achieves an average NMSE of 3.08% on the test set.

TABLE II

Model NMSE

| Device | Train | Test | NMSE |
| --- | --- | --- | --- |
| RDL | 400 | 100 | 0.04% |
| TSV | 400 | 100 | 0.02% |
| RDL-TSV | 120 | 58 | 3.08% |

The final average NMSE of 3.08% is slightly better than the accuracy reported in [2] using the same number of training samples.

The proposed model has two practical advantages. First, it is more compact than a direct full-chain S-parameter neural surrogate because it learns only the parameters of physically meaningful equivalent circuits. Second, the resulting model can be implemented in a SPICE-compatible form, making it suitable for compact-model library construction, circuit-level verification, and PDK-oriented development.

Conclusion

This work proposes a compact modeling method for cascaded RDL-TSV structures. By combining scalable RDL and TSV single-device models with an RDL-TSV Transition Model, the method builds an accurate high-dimensional compact model using limited full-wave data. A shared-to-multi-head training strategy reduces parasitic-parameter ambiguity while capturing position-dependent transition effects. For the TSV-RDL3 case, the model achieves 3.1% average NMSE with 120 training and 58 test samples. Since the final model is based on equivalent circuits, it more naturally preserves passivity and causality, supports SPICE simulation, and is suitable for PDK-oriented applications.

References

[1] Z.-X. Ye et al., "Frequency-Domain Modeling of Interconnects Based on Assemble Neural Network for 3-D Integration," IEEE Transactions on Computer-Aided Design of Integrated Circuits and Systems, vol. 45, no. 5, pp. 2208-2221, 2026.

[2] H.-S. Yin et al., "A SPICE-Compatible Model for On-Chip Coplanar Coupled Waveguides Up to 110 GHz," in 2024 IEEE 7th International Conference on Electronic Information and Communication Technology (ICEICT), Xi'an, China, 2024, pp. 412-415.

[3] Y. Zhang et al., "High-Frequency Transmission Characteristic Analysis of TSV-RDL Interconnects," IEEE Transactions on Components, Packaging and Manufacturing Technology, vol. 14, no. 1, pp. 89-97, Jan. 2024.